Instruction Manual
                     
[image: image1.png]Q. — A,

S o
= g30

HEEIEIEIE
U
=|Z|0o|a|o|x
EEIEIRIRIEIE
Waém—@wm
O |= =X |4~
EIEIEIRIRIEIE
— g
eo s [] (K] ST
<|o(O|0|W|w|n





Check that all parts and screws listed in the parts list are present (items marked A, B, C, D, E, F, G, H, I, J, K, L, M, N, O, P, Q, R, S，T).

Check the walls and the shower tray: 

-Floor must be level with a 1%-2% slope toward the drain to ensure proper water runoff. 

- Wall verticality tolerance should be ≤ 3mm; tile or stone surfaces must be fully cured and clean.

- Confirm drain location and ensure it is unobstructed to prevent water pooling.

Required tools:

drill with tile drill bit

screwdriver

spirit level

tape measure and pencil

silicone gun

(optional) rubber mallet

[image: image2.png]S—
T
~—
=
e,
—
[Pom
-
@0




 

1、Install the aluminum alloy frame: Fasten the upper rails and down rails A to the top and bottom of the vertical posts B on both sides respectively, and lock them with screws S. Shake the frame gently to test its stability and ensure there is no wobbling. Then install the stoppers P at the corresponding positions on the upper and down rails A (four pieces for each rail), and place the assembled frame aside for later installation use.

2、Measure the installation area accurately, use a tape measure to locate the drilling positions on the wall, mark the positions with a pencil, and use a level to ensure all marked lines are completely horizontal and vertical. Then drill holes with an electric drill, and fix the wall material C to the wall with the accessories M, N and R.

[image: image3.png]R—
1egre
stax12
By
0

[ o

stoxiz

=

[

e





3、Insert the frame assembled in Step 1 into C, adjust the frame at the proper location, then drill three holes inside the frame with an electric drill, and fasten B and C together using screws T and decorative caps R. (The same operation applies to both sides)

4、Insert the fixed glass panel D into frame B, clamp the glass clamp L onto the fixed glass panel D, mark the position on rail A with a pencil, drill a hole with an electric drill, fasten the glass clamp L to rail A with screws Q, and finally insert the rubber strips F into the gap between B and D. (The same operation applies to both sides)

[image: image4.png]


  
5、Install the sliding glass panels: 

-Mount the upper rollers H on the top of the sliding glass panel E, and install the lower roller I at the bottom of the sliding glass panel E. -Carefully slide the sliding glass panel with rollers installed into the upper and lower guide rails. Adjust the rollers height to ensure smooth, silent sliding without jamming. (The same operation applies to both sides)

[image: image5.png]G| [T |x





6、Install the waterproof strips and magnetic strips: Mount the waterproof strips G at the corresponding positions of the fixed glass and sliding glass panels, and correctly install the plastic magnetic strips J on the two sliding glass panels.

[image: image6.png]



7、Install the handle K on the two sliding glass panels.

8、Apply the glass sealant.

- Apply a neat bead of silicone sealant along all joints between glass and frame, and between frame and wall/floor. Smooth the sealant with a wet finger for a clean finish and enhanced waterproofing.

- Remove excess silicone sealant with a damp cloth.

- Test door operation repeatedly to ensure smooth sliding or swinging; check for any abnormal noise or resistance. 

- Conduct a water test: Run water inside the enclosure and check for leaks around the threshold, frame joints and door seal.

- Allow the sealant to fully cure for 24-48 hours before regular use.

Szerelési útmutató

**Ellenőrizze, hogy az alkatrészlistában felsorolt összes alkatrész és csavar megvan (A, B, C, D, E, F, G, H, I, J, K, L, M, N, O, P, Q, R, S，T jelölésű tételek).
**

Ellenőrizze a falakat és a zuhanytálcát:
-A padlónak vízszintesnek kell lennie, 1%-2% lejtéssel a lefolyó irányába, a megfelelő vízelvezetés biztosítása érdekében.

· A fal függőlegességi tűrése legyen ≤ 3mm; a csempe- vagy kőfelületeknek teljesen kikeményedettnek és tisztának kell lenniük.

· Ellenőrizze a lefolyó helyét, és győződjön meg róla, hogy nincs elzáródva, a víz felgyülemlésének megelőzése érdekében.

**Szükséges szerszámok:
**

fúró csempéfúró szárral

csavarhúzó

vízmérték

mérőszalag és ceruza

szilikonpisztoly

(opcionális) gumikalapács

1、Szerelje fel az alumíniumötvözet keretet: Rögzítse a felső és az alsó A sínt a B függőleges tartóoszlopok tetejére és aljára mindkét oldalon, és zárja le S csavarokkal. Rázza meg finoman a keretet, hogy ellenőrizze annak stabilitását, és győződjön meg róla, hogy nem billeg. Ezután szerelje fel a P ütközőket a megfelelő pozíciókba a felső és az alsó A sínen (minden sínhez négy darab), és tegye félre az összeszerelt keretet a későbbi beépítéshez.

2、Mérje fel pontosan a beépítési területet, mérőszalaggal jelölje ki a falon a fúrási pontokat, ceruzával jelölje meg a pozíciókat, és vízmértékkel ellenőrizze, hogy az összes jelölés teljesen vízszintes és függőleges legyen. Ezután fúrjon lyukakat elektromos fúróval, és rögzítse a C falra rögzülő anyagot a falra az M, N és R tartozékokkal.

3、Helyezze be az 1. lépésben összeszerelt keretet a C elembe, állítsa a keretet a megfelelő helyre, majd fúrjon három lyukat a kereten belül elektromos fúróval, és rögzítse a B és C elemeket egymáshoz T csavarokkal és R díszkapcsoló kupakokkal. (Ugyanez a művelet mindkét oldalon)

4、Helyezze be a D fix üveglapot a B keretbe, fogassa rá az L üvegfogót a D fix üveglapra, ceruzával jelölje meg a pozíciót az A sínen, fúrjon egy lyukat elektromos fúróval, rögzítse az L üvegfogót az A sínhez Q csavarokkal, végül pedig helyezze be az F gumicsíkokat a B és D közötti résbe. (Ugyanez a művelet mindkét oldalon)

5、Szerelje fel a csúszó üveglapokat:

-Szerelje fel a H felső görgőket az E csúszó üveglap tetejére, és szerelje fel az I alsó görgőt az E csúszó üveglap aljára. -Óvatosan csúsztassa be a görgőkkel felszerelt csúszó üveglapot a felső és alsó vezetősínekbe. Állítsa be a görgők magasságát a sima, csendes csúszás érdekében, anélkül hogy beszorulna. (Ugyanez a művelet mindkét oldalon)

6、Szerelje fel a vízzáró csíkokat és mágnescsíkokat: Szerelje fel a G vízzáró csíkokat a fix üveglap és a csúszó üveglapok megfelelő helyeire, és helyezze fel megfelelően a J műanyag mágnescsíkokat a két csúszó üveglapra.

7、Szerelje fel a K fogantyút a két csúszó üveglapra.

8、Vigye fel az üvegtömítő szilikont.

· Vigyen fel egyenletes szilikoncsíkot az üveg és a keret közötti, valamint a keret és a fal/padló közötti összes illesztés mentén. Nedves ujjal simítsa el a szilikont a tiszta felület és a jobb vízzárás érdekében.

· Távolítsa el a felesleges szilikont nedves ruhával.

· Többször ellenőrizze az ajtó működését a sima csúszás vagy nyitás érdekében; ellenőrizze, van-e rendellenes zaj vagy ellenállás.

· Végezzen vízpróbát: engedjen vizet a kabin belsejében, és ellenőrizze, hogy van-e szivárgás a küszöbnél, a keret illesztéseinél és az ajtótömítésnél.

· A normál használat előtt hagyja a tömítőanyagot 24-48 órán át teljesen kikeményedni.

