Instruction Manual


   [image: image1.png]R| < [x1
5

444444444

22222222

Al = X




      

Check that all parts and screws listed in the parts list are present (items marked A,B,C,D, E,F,G,H, I, J, K, L,M, N,O,P, Q1,Q2, R, S,T,U,V,W).

Check the walls and the shower tray: 

-Floor must be level with a 1%-2% slope toward the drain to ensure proper water runoff. 

- Wall vertical tolerance should be ≤ 3mm; tile or stone surfaces must be fully cured and clean.

- Confirm drain location and ensure it is unobstructed to prevent water pooling.

Required tools:

drill with tile drill bit

Screwdriver

Adjustable wrench
spirit level

tape measure and pencil

silicone gun

(optional) rubber mallet

[image: image2.png]



1、 Fasten the upper and lower rails A to the top and bottom of each vertical post B, and lock them with screws T. Then connect the left and right frames with 2 L-shaped connectorsQ1 (1 at the top and 1 at the bottom), and reinforce them with W accordingly. Gently shake the frame to test its stability and ensure there is no wobbling. 

2、 Insert the fixed glass panel D into frame B, clamp the glass clamp M onto the fixed glass panel D, mark the position on rail A with a pencil, drill a hole with an electric drill, fasten the glass clamp M to rail A with screws Q2, and finally insert the rubber strips J into the gap between B and D. (The same operation applies to both sides).And place the assembled frame with fixed glass aside for later installation use.

3、Mount the frame in step2 onto the shower tray.

4、Install mixer V and shower part S onto the function panel, connect the faucet outlet and water inlet with a plastic pipe on the back of the function panel.
[image: image3.png]



5、Install the shelf onto the rear glass panel F, and place it aside for later installation use.

6、Fasten rear panel with shelf F and the Frame in step2 with screws(4Pcs) I, and fasten rear panel without shelf G and the Frame with screws(4Pcs) I too. Fasten rear panel with shelf F, the function panel and the rear panel without shelf G with screws(4Pcs) I. Install the stoppers at the corresponding positions on the upper and down rails A (2 pieces for each rail).

7、Connect the adapter and the shower with shower hose.
[image: image4.png]


 
8、Install the sliding glass panels: 

-Mount the upper rollers N on the top of the sliding glass panel E, and install the lower roller Q at the bottom of the sliding glass panel N. -Carefully slide the sliding glass panel with rollers installed into the upper and lower guide rails. Adjust the rollers height to ensure smooth, silent sliding without jamming. (The same operation applies to both sides)

9、Drill the hole and fasten the aluminum of the rear glass panel to the shower tray with screws U.
10、Install the waterproof strips and magnetic strips: Mount the waterproof strips K at the corresponding positions of the fixed glass and sliding glass panels, and correctly install the plastic magnetic strips L on the two sliding glass panels.

10、 Install the handle P on the two sliding glass panels.

11、Apply the glass sealant.

- Apply a neat bead of silicone sealant along all joints between glass and frame, and between frame and wall/floor. Smooth the sealant with a wet finger for a clean finish and enhanced waterproofing.

- Remove excess silicone sealant with a damp cloth.

- Test door operation repeatedly to ensure smooth sliding or swinging; check for any abnormal noise or resistance. 

- Conduct a water test: Run water inside the enclosure and check for leaks around the threshold, frame joints and door seal.

- Allow the sealant to fully cure for 24-48 hours before regular use.
Szerelési útmutató

**Ellenőrizze, hogy az alkatrészlistában felsorolt összes alkatrész és csavar megvan (A,B,C,D, E,F,G,H, I, J, K, L,M, N,O,P, Q1,Q2, R, S,T,U,V,W jelölésű tételek).
**

Ellenőrizze a falakat és a zuhanytálcát:
-A padlónak vízszintesnek kell lennie, 1%-2% lejtéssel a lefolyó irányába, a megfelelő vízelvezetés biztosítása érdekében.

· A fal függőlegességi tűrése legyen ≤ 3mm; a csempe- vagy kőfelületeknek teljesen kikeményedettnek és tisztának kell lenniük.

· Ellenőrizze a lefolyó helyét, és győződjön meg róla, hogy nincs elzáródva, a víz felgyülemlésének megelőzése érdekében.

**Szükséges szerszámok:
**

fúró csempéfúró szárral

csavarhúzó

állítható villáskulcs

vízmérték

mérőszalag és ceruza

szilikonpisztoly

(opcionális) gumikalapács

Rögzítse a felső és az alsó A sínt minden egyes B függőleges tartóoszlop tetejére és aljára, és zárja le T csavarokkal. Ezután kösse össze a bal és a jobb keretet 2 L alakú Q1 csatlakozóval (1 a tetején és 1 az alján), és erősítse meg őket W elemekkel ennek megfelelően. Rázza meg finoman a keretet, hogy ellenőrizze annak stabilitását, és győződjön meg róla, hogy nem billeg.

Helyezze be a D fix üveglapot a B keretbe, fogassa rá az M üvegfogót a D fix üveglapra, ceruzával jelölje meg a pozíciót az A sínen, fúrjon egy lyukat elektromos fúróval, rögzítse az M üvegfogót az A sínhez Q2 csavarokkal, végül pedig helyezze be a J gumicsíkokat a B és D közötti résbe. (Ugyanez a művelet mindkét oldalon).És tegye félre az összeszerelt keretet a fix üveggel a későbbi beépítéshez.

3、Szerelje fel a 2. lépésben szereplő keretet a zuhanytálcára.

4、Szerelje fel a V keverőcsapot és az S zuhany részt a funkciólapra, kösse össze a csaptelep kifolyóját és a vízbevezetőt egy műanyag csővel a funkciólap hátoldalán.

5、Szerelje fel a polcot a hátsó F üveglapra, és tegye félre a későbbi beépítéshez.

6、Rögzítse a polcos hátsó F panelt és a 2. lépésben szereplő keretet I csavarokkal (4db), és rögzítse a polc nélküli hátsó G panelt és a keretet is I csavarokkal (4db). Rögzítse a polcos hátsó F panelt, a funkciópanelt és a polc nélküli hátsó G panelt I csavarokkal (4db). Szerelje fel az ütközőket a megfelelő pozíciókba a felső és az alsó A sínen (minden sínhez 2 darab).

7、Kösse össze az adaptert és a zuhanyt a zuhanytömlővel.

8、Szerelje fel a csúszó üveglapokat:

-Szerelje fel az N felső görgőket az E csúszó üveglap tetejére, és szerelje fel a Q alsó görgőt az N csúszó üveglap aljára. -Óvatosan csúsztassa be a görgőkkel felszerelt csúszó üveglapot a felső és alsó vezetősínekbe. Állítsa be a görgők magasságát a sima, csendes csúszás érdekében, anélkül hogy beszorulna. (Ugyanez a művelet mindkét oldalon)

9、Fúrjon lyukat, és rögzítse a hátsó üveglap alumíniumát a zuhanytálcára U csavarokkal.

10、Szerelje fel a vízzáró csíkokat és a mágnescsíkokat: Szerelje fel a K vízzáró csíkokat a fix üveglap és a csúszó üveglapok megfelelő helyeire, és helyezze fel megfelelően az L műanyag mágnescsíkokat a két csúszó üveglapra.

Szerelje fel a P fogantyút a két csúszó üveglapra.

11、Vigye fel az üvegtömítő szilikont.

· Vigyen fel egyenletes szilikoncsíkot az üveg és a keret közötti, valamint a keret és a fal/padló közötti összes illesztés mentén. Nedves ujjal simítsa el a szilikont a tiszta felület és a jobb vízzárás érdekében.

· Távolítsa el a felesleges szilikont nedves ruhával.

· Többször ellenőrizze az ajtó működését a sima csúszás vagy nyitás érdekében; ellenőrizze, van-e rendellenes zaj vagy ellenállás.

· Végezzen vízpróbát: engedjen vizet a kabin belsejében, és ellenőrizze, hogy van-e szivárgás a küszöbnél, a keret illesztéseinél és az ajtótömítésnél.

· A normál használat előtt hagyja a tömítőanyagot 24-48 órán át teljesen kikeményedni.

